## Scope: Applies to toys for children, toys being any product or material designed or intended for use in play by children of less than 14 years.

| Certificate Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Certificate of Conformity | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Certificate of Conformity is needed for any importer/manufacturer to evidence compliance with the requirements and should be carried by local certification body. The certificate of Conformity (CoC) will be issued by certification body (i.e. SGS Columbia is one of it) basing on testing reports, QMS certificate etc. documents. The eventual certificate will be valid for 3 years and is subject to annual follow up. |  |  |
| Safety Requirements |  |  |  |  |
| Mechanical and physical | Resolution 686 of 5 March 2018, NTC EN 71-1 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Shall comply with the NTC EN 71-1 mechanical and physical requirements. (acceptable equivalent standards: EN 71-1, ASTM F963, ISO 8124-1) |  |  |
| Flammability | Resolution 686 of 5 March 2018, NTC EN 71-2 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Shall comply with the NTC EN 71-2 Flammability requirements. (acceptable equivalent standards: EN 71-2, ASTM F963, ISO 8124-2) |  |  |
| * Migration of Certain Elements | Resolution 686 of 5 March 2018, NTC EN 71-3 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Shall comply with the NTC EN 71-3 Migration of Certain Elements requirements. (acceptable equivalent standards: EN 71-3) |  |  |
| *Phthalate content | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Toys for children under three years of age, the below six phthalates (BBP, DBP, DOP or DEHP, DNOP, DINP and DIDP) total content should not exceed 0.1%, reference to EN 14372. |  |  |
| Safety of Electronic Toy | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Shall comply with the EN 62115 Electronic Toy requirements. |  |  |
| Labeling requirement |  |  |  |  |
| Language(s) | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Spanish |  |  |
| Factory Name | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Factory name must be present |  |  |
| Importer Name | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Importer or authorized distributor in Colombia must be present on the label |  |  |
| Reference No. & Batch # | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Reference No. & Batch number must be present. |  |  |
| Age grading | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Minimum age must be clearly specified. |  |  |
| Warnings | Resolution 686 of 5 March 2018 Link: https://www.minsalud.gov.co/sites/rid/Lists/BibliotecaDigital/RIDE/DE/DIJ/resolucion-686-de-2018.pdf | Warnings must be present in Spanish language. Warnings must be in accordance to the age grading, the presence of small parts and use of the product. Graphical symbols for age warning in accordance with NTC EN 71-1. |  |  |
| EPR (Extended producer responsibility) |  |  |  |  |
| Gradual reduction of single-use plastic products | Law No 2232/2022 Link： https://www.minambiente.gov.co/documento-normativa/ley-2232-de-2022/ | Colombia’s Law No. 2232 of 2022 aiming to ensure that by 2030, most single-use plastics and containers placed on the Colombian market will be biodegradable, reusable or made with locally sourced post-consumer recycled materials. 14 categories of single-use plastics will be gradually banned, including: 1. Checkout bags (e.g., supermarket plastic bags) 2. Bags for newspapers, ads, dry-cleaning 3. Fruit/vegetable roll bags (unless for animal products) 4. Cups, lids, containers for beverages or takeout 5. Cutlery: knives, forks, spoons, plates, gloves 6. Straws and stirrers 7. Plastic balloon sticks 8. Party items: confetti, plastic tablecloths 9. Takeout food containers (non-prepackaged) 10. Plastic films for food wrapping 11. Cotton swab sticks 12. Dental floss picks 13.Fresh produce packaging (unless recyclable and food-safe) 14. Plastic labels and stickers Phase; Products Banned; Effective Date. Phase 1; 1, 2, 3, 6, 7, 11; July 7, 2024. Phase 2; 4, 5, 8, 9, 10, 12, 13, 1; July 7, 2030. Applies to non-banned single-use plastics, - mandatory recycled content: PET water bottles: 50% by 2025 → 90% by 2030; Other PET bottles: 20% by 2025 → 35% by 2030 → 60% by 2040; - Collection target: 50% by 2030; - No imported waste allowed to meet targets. |  |  |

PRICING AND ADDITIONAL NOTE :

-- Attributes being marked with “*” are not covered by basic package price, additional charge will be required if they are applicable.

** It is the suggested number of samples required for complete testing only. Sample size/raw materials and number of working days may vary depending on the actual sample’s size and the number of tests required for testing. Please consult your nearest laboratory for details. In order to keep sample size the minimum, SGS will inform the supplier the exact number of samples after sample review.

| ** No of Sample Required: |  |
| --- | --- |
| No. Of Samples Required For Complete Testing: |  | Generally, 6pcs, If sample or component is insufficient to schedule testing, it may be required to provide additional samples to schedule testing: - PHT test (if any) should be more than 0.1g. |  |
| No. Of Working Days For Complete Testing: |  | 7 – 9 |  |

| Review History |  |  |  |
| --- | --- | --- | --- |
| Review No | Review Date | Reviewed by | Changes |
| 0 |  |  | Initial Release |

The Company’s consulting services, including any review of product safety regulations and industry standards listed in product protocols (“Protocols”) are based upon information provided by client, Company’s know-how and on publicly available sources at the time the services were supplied. Client is responsible for providing any and all information about its product design, manufacture and other accurate information that may be required to complete the protocols. Client is responsible for his decision to make whatever use of the information contained in the Protocols and accepts all risks for any loss or damage of any kind that may occur hereof. In addition, client shall be sole responsible for all liabilities for injuries caused by defects in the design, manufacturing or labelling of their products. Protocols must be used as guidance only, as similar products may have different detailed test requirements depending on specific functions. Company disclaims any and all liability for the accuracy of any such publicly available information or any legal interpretation of such information.  SGS provides its services in a consulting capacity only and offers no legal opinion(s) herein.  The opinions provided by the Company are not a substitute for professional legal advice and client should seek legal review to ensure compliance with any applicable laws and regulations.

The Company’s consulting services are governed by and subject to the General Conditions of Service at www.sgs.com/terms _and_conditions.htm. ©2023 SGS SA.  All rights reserved.