Scope: Applies to toys for children, toys being any product or material designed or intended for use in play by children of less than 14 years.

| Safety Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| KC Certification | The National Agency for Technology and Standards Link: 국가기술표준원 > 정책 > 제품안전 > 어린이제품 > 안전인증 | Manufacturer or importer shall test toy samples by the approved lab, （factory inspection maybe needed for some kinds of toys.） then notify to the notification body with the issued test report by the approved lab after testing . Manufacturer or importer shall mark the received notification number and KC mark on their own product or package before importing or distributing. |  |  |
| Mechanical and Physical Properties. | Safety Confirmation Standard Annex 6-Toys, part 2 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Shall comply with the mechanical and physical requirements. |  |  |
| Flammability | Safety Confirmation Standard Annex 6-Toys, part 3 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Shall comply with the Flammability requirements. |  |  |
| Hazardous chemical materials-- Migration of certain elements | Safety Confirmation Standard Annex 6-Toys, part 4 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Heavy metal – Category 1: Dry, brittle, powder like of pliable materials; Category 2: Liquid or sticky materials; Category 3: Scrapped-off materials Element; Category I mg/kg; Category II mg/kg; Category III mg/kg. Aluminum; 5625; 1406; 70000. Antimony; 45; 11.3; 560. Arsenic; 3.8; 0.9; 47. Barium; 1500; 375; 18750. Boron; 1200; 300; 15000. Cadmium; 1.3; 0.3; 17. Chromium (III); 37.5; 9.4; 460. Chromium (VI); 0.02; 0.005; 0.053. Cobalt; 10.5; 2.6; 130. Copper; 622.5; 156; 7700. Lead; 13.5; 3.4; 160. Manganese; 1200; 300; 15000. Mercury; 7.5; 1.9; 94. Nickel; 75; 18.8; 930. Selenium; 37.5; 9.4; 460. Strontium; 4500; 1125; 56000. Tin; 15000; 3750; 180000. Organic Tin; 0.9; 0.2; 12. Zinc; 3750; 938; 46000. |  |  |
| Hazardous chemical materials-- Total Lead and Cadmium content | Safety Confirmation Standard Annex 6-Toys, part 4 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Lead in substrate - Less than 100 mg/kg (textiles, metal tips of writing instruments and functional components of electronic products are exempt) Lead in paint - Less than 90 mg/kg Cadmium content - Less than 75 mg/kg (textiles are exempt) |  |  |
| Hazardous chemical materials-- Nickel Release | Safety Confirmation Standard Annex 6-Toys, part 4 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Ni-release for metal parts - Less than 0.5 µg/cm2/week. |  |  |
| Hazardous chemical materials-- Phthalates | Safety Confirmation Standard Annex 6-Toys, part 4 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Phthalates – DEHP, DBP, BBP, DINP, DIDP, DNOP and DIBP content be less than 0.1% (sum). The scope of phthalates is extended to cover all synthetic resin products, irrespective of whether they are intended to be mouthed, including coatings on textile or leather. |  |  |
| Activity toys for domestic use | Safety Confirmation Standard Annex 6-Toys, Part 5 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Activity toys for domestic use shall comply with requirements in this part. |  |  |
| Experimental sets for chemistry and related activities | Safety Confirmation Standard Annex 6-Toys, Part 6 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Experimental sets for chemistry and related activities shall comply with requirements in this part. |  |  |
| Finger paint | Safety Confirmation Standard Annex 6-Toys, Part 7 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Finger paint shall comply with requirements in this part. |  |  |
| Organic chemical compounds | Safety Confirmation Standard Annex 6-Toys, Part 8~10 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | The standard specifies requirements for the migration or content of certain hazardous organic chemical compounds from/in toys. For the following substances requirements are set: Flame retardants Tri-o-cresyl phosphate: ≤ 50 mg/kg Tris(2-chloroethyl) phosphate: ≤ 50 mg/kg Tris(2-chloro-1-methylethyl) phosphate: ≤ 5 mg/kg Tris(1,3-dichloro-2-propyl) phosphate: ≤ 5 mg/kg Colourants Disperse Blue 1 : ≤ 10 mg/kg Disperse Blue 3 : ≤ 10 mg/kg Disperse Blue 106 : ≤ 10 mg/kg Disperse Blue 124 : ≤ 10 mg/kg Disperse Yellow 3 : ≤ 10 mg/kg Disperse Orange 3 : ≤ 10 mg/kg Disperse Orange 37/76 : ≤ 10 mg/kg Disperse Red 1 : ≤ 10 mg/kg Solvent Yellow 1 : ≤ 10 mg/kg Solvent Yellow 2 : ≤ 10 mg/kg Solvent Yellow 3 : ≤ 10 mg/kg Basic Red 9 : ≤ 10 mg/kg Basic Violet 1 : ≤ 10 mg/kg Basic Violet 3 : ≤ 10 mg/kg Acid Red 26 : ≤ 10 mg/kg Acid Violet 49 : ≤ 10 mg/kg Primary aromatic amines Benzidine : ≤ 5 mg/kg 2-Naphthylamine : ≤ 5 mg/kg 4-Chloroaniline : ≤ 5 mg/kg 3,3'-Dichlorobenzidine : ≤ 5 mg/kg 3,3'-Dimethoxybenzidine : ≤ 5 mg/kg 3,3'-Dimethylbenzidine : ≤ 5 mg/kg o-Toluidine : ≤ 5 mg/kg 2-Methoxyaniline(o-Anisidine) : ≤ 5 mg/kg Aniline : ≤ 5 mg/kg Monomers Acrylamide : ≤ 0.02 mg/L Bisphenol A : ≤ 0.1 mg/L Formaldehyde : ≤ 2.5 mg/L Phenol : ≤ 15 mg/L Styrene : ≤ 0.75 mg/L Solvents (migration) Trichloroethylene : ≤ 0.02 mg/L Dichloromethane : ≤ 0.06 mg/L 2-Methoxyethyl acetate + 2-Ethoxyethanol + 2-Ethoxyethyl acetate + Bis(2-methoxyethyl) ether + 2-Methoxypropyl acetate : ≤ 0.5 mg/L Methanol : ≤ 5 mg/L Nitrobenzene : ≤ 0.02 mg/L Cyclohexanone : ≤ 46 mg/L 3,5,5-Trimethyl-2-cyclohexene-1-one (Isophorone) : ≤ 3 mg/L Toluene : ≤ 2 mg/L Ethylbenzene : ≤ 1 mg/L Xylene (all isomers) : ≤ 2 mg/L (total) Solvents (inhalation) Toluene : ≤ 260 μg/m3 Ethylbenzene : ≤ 5,000 μg/m3 Xylene (all isomers) : ≤ 870 μg/m3 (total) 1,3,5-Trimethylbenzene (mesitylene) : ≤ 2,500 μg/m3 Trichloroethylene : ≤ 33 μg/m3 Dichloromethane : ≤ 3,000 μg/m3 n-Hexane : ≤ 1,800 μg/m3 Nitrobenzene : ≤ 33 μg/m3 Cyclohexanone : ≤ 136 μg/m3 3,5,5-Trimethyl-2-cyclohexene-1-one : ≤ 200 μg/m3 Preservatives / Wood preservatives Pentachlorophenol : ≤ 2 mg/kg 2,4-Dichlorophenol : ≤ 5 mg/kg 2,4,6-Trichlorophenol : ≤ 5 mg/kg 2,4,5-Trichlorophenol : ≤ 10 mg/kg 2,3,4,6-Tetrachlorophenol : ≤ 1 mg/kg Lindane : ≤ 2 mg/kg Cyfluthrin : ≤ 10 mg/kg Cypermethrin : ≤ 10 mg/kg Deltamethrin : ≤ 10 mg/kg Permethrin : ≤ 10 mg/kg Preservatives (other than wood) Phenol : 10 mg/kg 1,2-Benzylisothiazolin-3-one : ≤ 5 mg/kg 2-Methyl-4-isothiazolin-3-one : Prohibited 5-Chloro-2-methyl-4-isothiazolin-3-one : Prohibited 5-Chloro-2-methyl-4-isothiazolin-3-one + 2-Methyl-4-isothiazolin-3-one : Prohibited Plasticisers (excluding phthalates) Triphenyl phosphate : ≤ 0.03 mg/L Tri-o-cresyl phosphate : ≤ 0.03 mg/L Tri-m-cresyl phosphate : ≤ 0.03 mg/L Tri-p-cresyl phosphate : ≤ 0.03 mg/L Formaldehyde For textile and paper : ≤ 30 mg/kg For wood : ≤ 80 mg/kg Colorfastness (ISO 105-E04) Over than level 3 pH For liquid : 3.0 ~ 10.0 |  |  |
| Microbiological safety of toys containing aqueous media | Safety Confirmation Standard Annex 6-Toys, Part 11 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Aqueous media in toy should fulfil following requirement: Microbial; Limits (cfu/mL or cfu/g). Total Aerobic Microbial Count; 1000. Total Yeasts and Molds Count; 100. Staphylococcus aureus; Absence. Pseudomonas aeruginosa; Absence. Escherichia coli; Absence. |  |  |
| Total content of N-nitrosamines and N-nitrosatable substances in children’s products containing elastomeric materials | Safety Confirmation Standard Annex 6-Toys, part 4& part 12 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | N-nitrosamines and N-nitrosatable substances; CAS No.; Limit. Category I; Category II. N-Nitrosamines; NDMA; 62-75-9; Total ≤ 0.01 mg/kg; Total ≤ 0.05 mg/kg. NDEA; 55-18-5. NDPA; 621-64-7. NDBA; 924-16-3. NPIP; 100-75-4. NPYR; 930-55-2. NMOR; 59-89-2. N-Nitrosatable substances; NDMA; 62-75-9; Total ≤ 0.1 mg/kg; Total ≤ 1.0 mg/kg. NDEA; 55-18-5. NDPA; 621-64-7. NDBA; 924-16-3. NPIP; 100-75-4. NPYR; 930-55-2. NMOR; 59-89-2. Note: 1. Category I: elastomeric children’s products intended to be used in the mouth of children under 36 months old (e.g., soother, teats, toothbrushes) 2. Category II: a) Elastomeric-containing products for children under 36 months other than those in category I. b) Elastomeric children’s products intended to be used in or put into the mouth of children over 36 months old, for example, balloons and mouthpieces and similar products.. |  |  |
| Electronic Toy |  |  |  |  |
| Battery-Operated Toys | Safety Confirmation Standard Annex 6-Toys, Part 2， Appendix A Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Battery-Operated Toys should comply with the requirements in this section. |  |  |
| EMC & EMI & EMS | Radio Waves Act Link: 국가법령정보센터 ¦ 영문법령 > 본문 - 전파법 | EMC - Comply with Electromagnetic Compatibility for battery operated toys - KSC 9814-1, 2 EMI & EMS- Comply with Electromagnetic Interference for radio-controlled toys - EMI: KS C 9814-1 - EMS: KS C 9814-2 |  |  |
| Electrical Safety Compliance | Electric Appliance Safety Control Act Link: Statutes of the Republic of Korea | Shall comply with the applicable technical requirements and have valid test report and test certificate. [Vendor shall provide valid KC-mark certificate and test report issued not more than 2 years old for document review] |  |  |
| Labelling Requirements |  |  |  |  |
| Country of Origin Marking (Applicable for goods other than pharmaceutical, food and agricultural products) | Foreign Trade Act (Republic of Korea) Link: 국가법령정보센터 ¦ 영문법령 > 본문 - 대외무역법 | Country of origin labeling shall be provided. |  |  |
| Safety Confirmation Standard Annex 6 ‘Toy’ | Korea Agency for Technology and Standards a Notification No. 2022-0221 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | Labelling requirements: - The KC-mark with notification no. shall be marked, and the size can be adjusted depends on the product size. - The name and/or trade name and/or mark and address of the manufacturer or its authorized representative or the importer in Korean on toy. - In the case of small toys and toys consisting of small parts these particulars may be affixed to the minimum packaging, to label or to a leaflet. |  |  |
| Safety Confirmation Standard Annex 6 ‘Toy’ | Korea Agency for Technology and Standards a Notification No. 2022-0221 Link: 국가기술표준원 > 정책 > 제품안전 > 안전기준열람 > 어린이제품안전기준 | - Followings shall be labelled on package in Korean 1. Product Type 2. Product name 3. Manufacture date 4. Manufacture Name 5. Import Name (if applicable) 6. Address of manufacture or the importer 7. Use ages 8. Limited weight of children to use (if applicable) 9. Origin of manufacture - Applicable warning clauses as detailed in KC Annex 6 Toy part1 - Advise to keep information for reference (if applicable) - Toys not intended for children under 36 months (if applicable) "not suitable for children under 36 months" or "not suitable for children under 3 years" and a brief indication, which may also appear in a leaflet or the instructions for use. Should comply with the labeling requirements of KC Annex 6 Toy part1 - Toy contained DNOP, DINP and DIDP shall be marked on toy or package as “Warning! Phthalate could be migrated when placed in the mouth. Don’t put this toy in mouth!” |  |  |
| Labeling (For radio controlled item only) | Korea Radio Research Agency. Link: National Radio Research Agency | Shall meet the labeling requirements as follows 1. KCC ID ”R” (1 character) 2. Authentication field identification code certification C (certification) Registration R (registration) Provisional certification I (interim) 3. Testing field identification code Wireless R (Radio / wireless) wired T (Telecommunication) Electromagnetic E (Electromagnetic) Multi M (Multi function) 4. Certificate applicant identification code Manufacturer M (Manufacturer) Importer I (Importer) Seller S (Seller) 5. Company Code (3 digits in alphabetic words or numeric numbers, issued by RRA) 6. Product ID (14 digits maximum) to be assigned by the applicant composed of Alphabetic words, Numeric numbers, Hyphen (-) and Underscore (_) Note: - Marking on packaging can be omitted if use the transparent packaging material to check the conformity assessment marking (KC mark with identification code) of the product in packaging. - If the maximum sectional area is less than 400mm2 that hard to label the conformity assessment marking on the product, then marking attached to the packaging or basic design or identification code on the product are allowed - When purchasing parallel import goods, if the conformity assessment marking sticker is provided and purchaser can stick it directly on the product surface, the conformity assessment marking on the product can be omitted. |  |  |
| EPR (Extended producer responsibility) |  |  |  |  |
| Korea Radioactive Materials Testing | Act on Protective Action Guidelines Against Radiation in the Natural Environment | Korea bans radioactive material which contains a natural radionuclide, such as Uranium 235, Uranium 238, Thorium 232, and nuclides in radioactive decay series, in consumer products that are worn on or used in the close contact with human body. |  |  |

PRICING AND ADDITIONAL NOTE :

** It is the suggested number of samples required for complete testing only. Sample size/raw materials and number of working days may vary depending on the actual sample’s size and the number of tests required for testing. Please consult your nearest laboratory for details. In order to keep sample size the minimum, SGS will inform the supplier the exact number of samples after sample review.

| ** No of Sample Required: |  |
| --- | --- |
| No. Of Samples Required For Complete Testing: |  | 6 |  |
| No. Of Working Days For Complete Testing: |  | 7 – 9 |  |

| Review History |  |  |  |
| --- | --- | --- | --- |
| Review No | Review Date | Reviewed by | Changes |
| 0 |  |  | Initial Release |

The Company’s consulting services, including any review of product safety regulations and industry standards listed in product protocols (“Protocols”) are based upon information provided by client, Company’s know-how and on publicly available sources at the time the services were supplied. Client is responsible for providing any and all information about its product design, manufacture and other accurate information that may be required to complete the protocols. Client is responsible for his decision to make whatever use of the information contained in the Protocols and accepts all risks for any loss or damage of any kind that may occur hereof. In addition, client shall be sole responsible for all liabilities for injuries caused by defects in the design, manufacturing or labelling of their products. Protocols must be used as guidance only, as similar products may have different detailed test requirements depending on specific functions. Company disclaims any and all liability for the accuracy of any such publicly available information or any legal interpretation of such information. SGS provides its services in a consulting capacity only and offers no legal opinion(s) herein. The opinions provided by the Company are not a substitute for professional legal advice and client should seek legal review to ensure compliance with any applicable laws and regulations. The Company’s consulting services are governed by and subject to the General Conditions of Service at www.sgs.com/terms _and_conditions.htm. ©2023 SGS SA. All rights reserved.