## Scope: Applies to toys for children, toys being any product or material designed or intended, whether or not exclusively, for use in play by children not over 14 years of age.

| Safety Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Thailand |  |  |  |  |
| *Prohibited Materials | TIS 685 Part 1-2562 (2019) Section 3 Link to TIS 685 Part 1-2562 | The following types of materials are prohibited in the making of toys: - Materials with abnormal expansion upon contact with water. - Testing methods must comply with TIS 685 Part 2 Clause 3.23. - Celluloid or other materials exhibiting similar flammability properties to celluloid. - Exceptions apply to celluloid used in paints, adhesives, varnishes, table tennis balls, or other similar sports balls. - Materials with surface flash effects when in proximity to flames. - Highly flammable solids. - Flammable gases, gels, and liquids. - Substances that spontaneously generate heat or ignite at room temperature. - Substances that release flammable gases upon contact with water or atmospheric moisture, such as calcium carbide. - Glass. - Restricted for toys intended for children younger than 3 years. Toys for children aged 3 years and above may use glass only when it is essential, such as marbles or parts integral to the toy's functioning. - Wood preservatives or treatments that are toxic or harmful to health. - As prescribed in the Hazardous Substance Regulations issued by the Ministry of Industry. |  |  |
| Mechanical and physical properties | TIS 685 Part 1-2562 (2019) Section 4.1 – 4.4 Link to TIS 685 Part 1-2562 | Shall comply with the physical and mechanical requirements in section 4.1 to 4.4 of the standard. |  |  |
| Flammability | TIS 685 Part 1-2562 (2019) Section 4.5 Link to TIS 685 Part 1-2562 | Shall comply with the flammability requirements in section 4.5 of the standard. |  |  |
| *Migration of certain elements | TIS 685 Part 1-2562 (2019) Section 4.6.1 / TIS 685 Part 2-2562 (2019) Section 3.39 Link to TIS 685 Part 1-2562 | Migration of certain elements from toys and toy materials must not exceed the limits in below table: Maximum limit for elements in the solution, extracted from toys and toy-making (milligrams / kilogram). Coatings, writing materials, plastics, paper, cardboard and textiles; Clay materials; Finger paint. Antimony (Sb); 60; 60; 10. Arsenic (As); 25; 25; 10. Barium (Ba); 1,000; 250; 350. Cadmium (Cd); 75; 50; 15. Chromium (Cr); 60; 25; 25. Lead (Pb); 90; 90; 25. Mercury (Hg); 60; 25; 10. Selenium (Se); 500; 500; 50. |  |  |
| *Phthalates | TIS 685 Part 1-2562(2019) Section 4.6.2 / ISO 8124-6 Link to TIS 685 Part 1-2562 | Total content of phthalates extracted from plastics, rubbers and coatings in the exposed area or part must not exceed the limits in below table: The maximum threshold for the total quantity of phthalate derivatives in the test specimens extracted from plastics, rubber and coatings in exposed areas or parts. The unit is a percentage fraction by mass.. General toys; Toys for children under 3 years old and have parts that can be put in the mouth. bis-(2-ethylhexyl) phthalate (DEHP); Not exceed 0.1%; Not exceed 0.1%. di-n-butyl phthalate (DBP). benzyl butyl phthalate (BBP). di-iso-nonyl phthalate (DINP); /. di-iso-decyl phthalate (DIDP). di-n-octyl phthalate (DNOP). |  |  |
| TISI license | Industrial Product Standards Act, B.E. 2511 (1968) and its amendment Link to B.E. 2511 (1968) and its amendment | The manufacturer or importer of toys should apply for a TISI license. https://www.tisi.go.th/ Procedure of apply a license: tisi.go.th/data/stories/license/pdf/PIL_en.jpg |  |  |
| Radio Equipment (If applicable) | NBTC | Radio Equipment should be certificated for NBTC, when applicable. https://www.nbtc.go.th/ |  |  |
| Indonesia |  |  |  |  |
| Mechanical and physical properties | SNI ISO 8124-1:2010 Link to SNI ISO 8124-1:2010 | Shall comply with the physical and mechanical requirements. |  |  |
| *Flammability | SNI ISO 8124-2:2010 Link to SNI ISO 8124-2:2010 | Shall comply with the flammability requirements. |  |  |
| *Migration of certain elements | SNI ISO 8124-3:2010 Link to SNI ISO 8124-3:2010 | Maximum acceptable levels for the migration of the elements antimony, arsenic, barium, cadmium, chromium, lead, mercury and selenium from toy materials and from parts of toys, except materials not accessible Toy Material; Requirements(mg/kg). Sb; As; Ba; Cd; Cr; Pb; Hg; Se. applicable material other than modelling clay and finger paint; 60; 25; 1000; 75; 60; 90; 60; 500. modelling clay and finger paint; 60; 25; 250; 50; 25; 90; 25; 500. |  |  |
| *Swings, slides and similar activity toys for indoor and outdoor family domestic use | SNI ISO 8124-4:2010 Link to SNI ISO 8124-4:2010 | Shall comply with the requirements of SNI ISO 8124-4. |  |  |
| *AZO dyes | SNI 7617:2010 Link to SNI 7617:2013, SNI 7334:2009 | Not used. (≤ 20 ppm) Note: Applicable for Toys that made from fabric |  |  |
| *Formaldehyde | SNI 7617:2010 Link to SNI 7617:2013 | Maximum 20ppm Note: Applicable for Toys that made from fabric |  |  |
| *Phthalates | EN 71-5 / CPSC-CH-C1001-09.4 | TOTAL PHTHALATE ≤ 0.1% Note: For Phthalates the test method is using CPSC CH-C101-09 for DEHP, DBP, BBP, DINP, DIDP and DNOP, with limit ≤0.1% (as alternative) |  |  |
| *Electrical toys | SNI IEC 62115:2011 Link to SNI IEC 62115:2011 | Must meet the requirement of SNI IEC 62115:2011. |  |  |
| *Mandatory SNI certification | Ministry of Industry Decree No.55/M-IND/PER/11/2013, Amendment of Regulation Ministry of Industry No. 24/MIND/PER/4/2013 | Registration to MoT is required for all toys sold in Indonesia. This must be done AFTER obtaining the SNI certification and before any shipment to Indonesia. The registration should be applied by local Importer and the registration number called NPB “Nomor Pendaftaran Barang” need to be printed out on SNI Label Hologram. https://akses-sni.bsn.go.id/ Links to Peraturan Menteri Perindustrian Nomor 55/M-IND/PER/11/2013, Peraturan Menteri Perindustrian Nomor 24/M-IND/PER/4/2013 |  |  |
| *Radio Equipment (If applicable) | DJID | Radio Equipment should be certificated for DJID, when applicable. https://www.postel.go.id/ |  |  |
| Labelling Requirements |  |  |  |  |
| Thailand |  |  |  |  |
| Toy Packaging | TIS 685 Part 1-2562(2019) Section 5 Link to TIS 685 Part 1-2562 | The toys must be packed in a clean, strong container that protects against damage that may occur to the toys inside. Packaging made from flexible plastic or plastic bags without a lining and wider than 100 mm must comply with section 4.3.7 and must display the messages "This packaging is not a toy" and "Do not place on head." Packaging that is a box or has a lid must be easy to open and must not contain any parts or materials that could be hazardous. |  |  |
| General Label | TIS 685 Part 1-2562(2019) Section 6.1~6.5 Link to TIS 685 Part 1-2562 | 1. The product packaging or containers must have a label that includes information in Thai language that is easy to read and clear, such as (1) the name of the manufacturer or the production plant and its location, or trademark registered; (2) the age of the player or age restriction according to item 6.6.1; (3) warnings according to item 6.6; (4) month and year of production. 2. The toys or containers for toys that are imported and sold domestically must have at least the following information clearly displayed in Thai (1) Name of the importer and location or trademark registration (2) Name of the manufacturer or factory and the country of manufacture (3) Age of the user or age limitations for the user according to Section 6.6.1 (4) Warning according to Section 6.6 (5) Month and year of manufacture or import. 3. In the case of goods intended for export, the labeling must comply with the agreement between the producer and the buyer. 4. The numbers and characters used must be at least 1 mm high, except for the details according to items 6.1(2), 6.1(3), 6.2(3), and 6.2(4) which must be at least 3 mm high. The word 'Warning' should be written in bold letters and must be at least 4 mm high. 5. The display colors of the background label and the colors of the details on the label according to items 6.1(2), 6.1(3), 6.2(3), and 6.2(4) must use contrasting colors that make the details clearly readable. |  |  |
| Warning and labeling of certain toy | TIS 685 Part 1-2562 (2019) Section 6.6~6.8 Link to TIS 685 Part 1-2562 | The toy should present appropriate warning and labeling specified in section 6.6 to 6.8 of the standard, as applicable. |  |  |
| Conformity Mark | Trademark Act B.E. 2559 (2016) | The toy and/or packaging should have appropriate TIS mark with Thailand standard number. Link to https://www.tisi.go.th/ |  |  |
| NBTC label (If applicable) | NBTC certification | Products should present appropriate certificate information, when applicable. https://www.nbtc.go.th/ |  |  |
| Indonesia |  |  |  |  |
| Labels in the Indonesian language | Regulation of the minister of trade of the republic of Indonesia Number 25 of 2021 Link to: Permendag No. 25 Tahun 2021, Nomor 67/M-DAG/PER/11/2013 | Below labelling for toys should be provided in Indonesian language. DESCRIPTION; LABEL PLACEMENT. Product; Packaging. a; Product name;; -; a/b/c/d. b; Brand;; a/b/c/d; a/b/c/d. c; Type of material or composition;; -; a/b/c/d. d; Name and address of manufacturer/retailer for local product; or Name and address of importer for import;; -; a/b/c/d. e; Product specification;; -; a/b/c/d. f; Maintenance label (care label) depends on type product;; -; a/b/c/d. g; Country of Origin or Made in.; a/b/c/d; a/b/c/d. h; Age grading; -; a/b/c/d. Notes: a. emboss; b. printed; c. completely attached; d. attached or included inside *) if label cannot attach on product, can attach on the packaging |  |  |
| Manufacturer’s markings | SNI ISO 8124-1:2010, Annex C4 Link to SNI ISO 8124-1:2010 | A principal component of the toy, the packaging, a label or a leaflet accompanying the toy should be marked with the name and address of the manufacturer or the distributor, or with a trademark and/or mark which clearly identifies the manufacturer / distributor. All these markings should be visible and easily legible by the consumer and should resist normal use conditions. |  |  |
| SNI ISO 8124 Labeling | SNI ISO 8124-1:2010 SNI ISO 8124-2:2010 SNI ISO 8124-4:2010 Link to SNI ISO 8124-1:2010, SNI ISO 8124-2:2010, SNI ISO 8124-4:2010 | Shall comply with the labeling requirement as specified. |  |  |
| SNI IEC 62115 LABELING | SNI IEC 62115:2011 Link to SNI IEC 62115:2011 | Shall comply with the labeling requirement as specified. |  |  |
| Conformity Mark | Ministry of Industry Decree No.55/M-IND/PER/11/2013, Amendment of Regulation Ministry of Industry No. 24/M- IND/PER/4/2013 Links to 02/BIM/PER/1/2014 | Appropriate SNI Mark with certificate information should present on the product and/or packaging as applicable. |  |  |
| DJID label (if applicable) | DJID certification | Products should present appropriate certificate information, when applicable. https://www.postel.go.id/ |  |  |
| EPR (Extended producer responsibility) |  |  |  |  |
| Thailand |  |  |  |  |
| * EPR (Extended producer responsibility) | the Product Eco-responsibility Ordinance | In order to strengthen the re-use and the prevention, recycling and other recovery of waste, producers bear the responsibility for the collection and treatment of their products when they reach end-of-life. Action Plan on Plastic Waste Management Phase ll (2023-2027) Draft the Act on Sustainable Packaging Management |  |  |
| Indonesia |  |  |  |  |
| * EPR (Extended producer responsibility) | the Product Eco-responsibility Ordinance | In order to strengthen the re-use and the prevention, recycling and other recovery of waste, producers bear the responsibility for the collection and treatment of their products when they reach end-of-life. UU No. 18 Tahun 2008 PP No. 81 Tahun 2012 PERPRES No. 97 Tahun 2017 PERPRES No. 83 Tahun 2018 jdih.menlhk.go.id/new2/home/portfolioDetails/75/2019/4 |  |  |

PRICING AND ADDITIONAL NOTE :

** It is the suggested number of samples required for complete testing only. Sample size/raw materials and number of working days may vary depending on the actual sample’s size and the number of tests required for testing. Please consult your nearest laboratory for details. In order to keep sample size the minimum, SGS will inform the supplier the exact number of samples after sample review.

| ** No of Sample Required: |  |
| --- | --- |
| No. Of Samples Required For Complete Testing: |  | 6 |  |
| No. Of Working Days For Complete Testing: |  | 7 – 9 |  |

| Review History |  |  |  |
| --- | --- | --- | --- |
| Review No | Review Date | Reviewed by | Changes |
| 0 |  |  | Initial Release |

The Company’s consulting services, including any review of product safety regulations and industry standards listed in product protocols (“Protocols”) are based upon information provided by client, Company’s know-how and on publicly available sources at the time the services were supplied. Client is responsible for providing any and all information about its product design, manufacture and other accurate information that may be required to complete the protocols. Client is responsible for his decision to make whatever use of the information contained in the Protocols and accepts all risks for any loss or damage of any kind that may occur hereof. In addition, client shall be sole responsible for all liabilities for injuries caused by defects in the design, manufacturing or labelling of their products. Protocols must be used as guidance only, as similar products may have different detailed test requirements depending on specific functions. Company disclaims any and all liability for the accuracy of any such publicly available information or any legal interpretation of such information. SGS provides its services in a consulting capacity only and offers no legal opinion(s) herein. The opinions provided by the Company are not a substitute for professional legal advice and client should seek legal review to ensure compliance with any applicable laws and regulations. The Company’s consulting services are governed by and subject to the General Conditions of Service at www.sgs.com/terms _and_conditions.htm. ©2023 SGS SA. All rights reserved.